Nakshatra Rane                                                                         AIEC1 Batch B

Roll No:18                                                                                  Exp2

- Setup Enviornment

mkdir blynk_dht11 && cd blynk_dht11

python3 -m venv --system-site-packages venv

source venv/bin/activate

pip install blynk-library-python adafruit-circuitpython-dht

- Blynk_dht11.py

import BlynkLib

import board

import adafruit_dht

import time

BLYNK_AUTH = '0aQ7ypo4j-OCPek4-fwQ7Phq1Wxxxxxx'

blynk = BlynkLib.Blynk(BLYNK_AUTH, server='blynk.cloud', port=80)

sensor = adafruit_dht.DHT11(board.D4)

print("Starting Blynk DHT11 Monitoring on Pi 5...")

def send_sensor_data():

try:

temp = sensor.temperature

humi = sensor.humidity

if humi is not None and temp is not None:

print(f"Blynk Update -> Temp: {temp:.1f}°C | Hum: {humi:.1f}%")

blynk.virtual_write(0, temp)

blynk.virtual_write(1, humi)

else:

print("Sensor returned null reading...")

except RuntimeError as error:

pass

except Exception as error:

sensor.exit()

raise error

last_send_time = 0

while True:

blynk.run()

current_time = time.time()

if current_time - last_send_time > 5.0:

send_sensor_data()

last_send_time = current_time

- Run the files

source venv/bin/activate

python3 blynk_dht11.py